2014. 2. 25

2014. 2. 25

Priority Setting for the Disaster Risk reduction on the Small Stream Watershed Regions to adapt Climate change

Priority Setting for the Disaster Risk reduction on the Small Stream Watershed Regions to adapt Climate change

Input Paper

Input Paper

Prepared for the Global Assessment Report on Disaster Risk Reduction 2015

Prepared for the Global Assessment Report on Disaster Risk Reduction 2015

Tae Sung Cheong

National Disaster Management Institute

Tae Sung Cheong

National Disaster Management Institute

Table of Contents

Introduction	3

Features and Current Issues on the SSWRPs	6

Multi Criteria Decision Analysis Modelling on the SSWRPs	7

Results	14

Official Priority Ranking Predicted by the Value Measurement Model	14

Priority Ranking Predicted by the Outranking Approach 	15

Evaluations of Multi Criteria Decision Approaches on the SSWRPs	17

Development of the Guideline for Selecting the Optimum SSWRPs	18

Conclusions	19

References	20

Image 1 : Comparison of total length of streams in Korea in which number describes number of each streams and percent represents restoration ratio of each streams	3

Image 2 : Comparison of damages with implementation in small streams	4

Image 3 : Concept diagram of the entropy weight calculation procedure	11

Image 4 : Concept diagram of the PROMETHEE procedure	13

Image 5 : Comparisons of determined priority ranking based on flood risk assessment with official ranking data used in verification	16

Image 6 : The results of the investment priorities, the positive outranking flow, the negative outranking flow and the net outranking flow	17

Image 7 : Comparison of discrepancy ratios of the PROMETHEE with six preference functions for official priority ranking used in verification	18

Table 1 : Brief description of three general strategies for prioritizing small stream restoration projects	7

Table 2 : The priority of the river restoration project on improving the flood prone area	8

Table 3 : Statistical chart of criteria collected from small steam watersheds in Korea	9

Table 4 : The indicators, sub indicators and criteria to set the priority of the SSWRPs ; 1Residential Area : Residential Area/Total Watershed Area; 2Restoration Ratio : Restoration Length/Total Channel Length	10

Table 5 : Six types of preference functions for the PROMETHEE method	12

Table 6 : Categorization of alternates for comparison between the evaluation criteria and the net outranking flow values	15

Table 7 : The Correlation coefficient of each criteria to the net outranking flow values	15

Table 8 : Guideline for the restoration measures for the flood related disaster risk management	19

# Introduction

Now, high intensity localized heavy rainfall is increasing in the pacific region which can cause massive flash floods making enormous damages especially in the small stream watersheds and even in the urban areas. Floods depend on precipitation intensity, volume, timing, antecedent conditions of streams and their drainage basins (Collins and Simpson, 2007). Human encroachment into flood plains and lack of flood response plans increase the damage potential. Development activity, particularly deforestation and intensive crop production, as well as increasing impervious cover may drastically change runoff conditions, thereby increasing stream flow during normal rainfall cycles and thus increasing the risk of flooding and soil degradation (Klein 1979; Bannerman et al. 1993; Coles et al. 2012). Intensive use of the floodplain, even under strict management, usually results in increased runoff rates. The small streams have a high risk for flash floods due to the presence of slopes thus, the rate of down cutting are high and the valley walls are often exposed to bedrock. Those in the mountains and steep narrow valleys generate rapid flowing waters which quickly raises water depths and thus resulting to a reduced evacuation time for residents in the affected areas and increased damages. In addition, streams such as those in low water crossings, mountains and urban areas rampant with pavement and roof covers have a high risk for flash floods. Flood-related disasters in Korea are classified according to several considerations like location, however, urbanized cities situated within the vicinity of small streams have become more susceptible and vulnerable to flash floods.

Among total streams of 26,558 (65,414 km) in Korea, 88 (3,276 km) of which are national streams whereas 3,858 streams (26,981 km) are local streams, both covered by the River Law (Han River Flood Control Office, 2012) shown in Image 1. The small streams managed by the Small River Maintenance Act are 22,612 streams (35,157 km) which is 53.7% of the total length of streams. The restoration lengths of the national, local small streams were 3,199km (98.2%), 18,417km (68.3%) respectively and the restoration lengths of local government managed small stream was 4,769km (42.0%). The 14,769 km small streams out of the total length of 35,157 km were restored 16 years from 1995 to 2012 and the total allocated budget for these projects was almost 310 billion US dollars.

Image 1 : Comparison of total length of streams in Korea in which number describes number of each streams and percent represents restoration ratio of each streams

Image 1 : Comparison of total length of streams in Korea in which number describes number of each streams and percent represents restoration ratio of each streams

The restoration ratio of the small streams managed by local government is not high because the lengths of these small streams is long and priority of them is relatively lower than national and local streams. The comparative results of damages in the smack streams and the Small Stream Restoration Projects (SSWRPs) budget from 2000 to 2012 demonstrated convincingly that the SSWRPs can help to reduce the flood-related damages in the small stream watersheds shown in Image 2.

Image 2 : Comparison of damages with implementation in small streams

Comparisons of damages with implementation in small streams

Image 2 : Comparison of damages with implementation in small streams

Comparisons of damages with implementation in small streams

Despite its success, existing small streams restoration projects were facing significant resource allocation challenges as well as problems for selecting optimum measures for the flood risk reduction considering community needs and available resources. Thus, the priority setting based on the flood risk assessment is needed for decision-making to allocate resources and select optimum community based restoration measures.

Priority setting approach based on the flood risk assessment can be used for defining the relative importance of the SSWRPs which can be used to protect sensitive watershed resources or restore small streams watershed for reducing flood risk and resilient recovering (Lee et al., 2011; Cheong et al., 2011). Lim et al. (2010) suggested representative factors for the disaster risk assessment constructed by P-S-R (Pressure-State-Response) classification system for identifying the watershed. UNISDR (2004) reported that hazards, vulnerabilities and capacity of local communities should be considered for the flood-related disaster risk assessment. Substantial progress has been made for the last decade regarding the development of the theoretical frameworks and practical strategies to guide and evaluate priority setting on various areas including flood control projects, water and sanitation improvement (Choi et al., 2013; Kang and Kim, 2006), multi-object dam construction and small stream watershed restoration (Mitton and Donaldson, 2003; Gibson et al., 2004; Roni et al, 2002; Pess et al., 2005; Beechie et al., 2000; Beechie, 2001). Ridgley (1992) used analytic hierarchy process method to define the ripple effects of drought on the water demand and supply and also presented analytic hierarchy process based on optimal water allocation method for reducing drought damages (Ridgley, 1993). Bruen (2002) validates both methods of analytic hierarchy process and multi-attribute utility theory to develop the watershed management decision support system and Jandric and Srdjevic (2000) used analytic hierarchy process and multi-attribute utility theory to find the optimal location for groundwater storage at NoviSad city in Yugoslavia. Jaber and Mohsen (2001) applied an analytic hierarchy process to find optimal solution on the water supply system. Udias et al. (2011) used A Multi-Objective Evolutionary Algorithm (MOEA) to identify efficient trade-offs between program of measures cost and water quality for selecting the most effective combinations of sewage treatment technologies. Raju et al. (2000) used the PROMETHEE and Elimination and Choice Translating Reality English - Elimination Et Choix Traduisant la Realite (ELECTRE) to evaluate the possibility of developing irrigation areas on Flumen Monegros in Spain. Also, the PROMETHEE was used for water resources management, optimal flood mitigation plan and priority setting of river restoration (Hyde et al., 2004; Maragoudaki and Tsakiris, 2005; Hermans et al., 2007). Khelifi et al. (2006) developed a decision support tool of Credence Clearwater Revival using the PROMETHEE II algorithm to assess the available technologies and select the preferred groundwater remedial options.

In Korea, most of the multi-criteria decision-making methods includes PROMETHEE are limitedly applied in operation level but actively used for research purposes. Within these methods, analytic hierarchy process method is widely used in integrated water resources management (Kim et al., 2012). Park (2002) applied analytic hierarchy process method to set the priority of current streams for adjustment of stream grade and enhancement of the efficiency of managing the river. For making decisions on the drought management, decision tree method was developed (Kang and Yoon, 2002; Kim, 2001). Korea Water Resources Corporation (2002) and Lee and Shim (2002) used an analytic hierarchy process to determine the weight and priority of water allocation. Kim et al. (2009) performed priority setting research to determine optimal location for development of small hydropower using the analytic hierarchy process method. Recently, Hong et al. (2006) performed risk assessment for the water distribution network using both the PROMETHEE and the analytic hierarchy process method. Kim (2006) proposed the new multi-criteria decision making methods for priority setting of dam construction projects by using analytic hierarchy process to determine the weights and added ELECTREE as a rank preferred technique. The PROMETHEE was used for flood inundation risk assessment (Choi, 2009) for increasing flood control capacities of the multi-objective reservoirs (Nam, 2007) and for priority setting of non-point pollutants management methods (Min, 2009). Park et al. (2005) and Kim et al. (2012) applied multi-attribute utility theory for priority setting of government-funded research and development projects. However, despite current efforts to create a comprehensive approach for priority setting, there are no single or easy tools that can help government officials to evaluate SSWRPs and therefore guide them to select optimal measures for flood-related disaster risk reduction.

In here, the 12 representative factors for the flood risk assessment are carefully selected and constructed for the three main aspects under the P-S-R classification system for identifying 212 small streams watershed. The entropy weight coefficient method is used to determine unbiased weight of criteria. The official priority ranking on the SSWRPs is predicted by the weighted method. The priority ranking of the flood risk assessment estimated by the PROMETHEE with six alternative preference functions are compared with the official values to select best preference function. The priority ranking estimated by proposed method are divided by three groups such as low priority group for prevention or preparedness projects, middle priority group for sustainable development project and high priority group for resilient recovering projects which can be used for decision-making to find local adapted SSWRPs for  protecting sensitive watershed resources or restoring small streams. Also, this will serve as guidelines to establish measures for reducing flood-related damages due to localized heavy rainfall and massive flash flooding.

# Features and Current Issues on the SSWRPs

In Korea, the main goal of the SSWRPs is to recover stream channels to its natural states by means of sustaining hydraulic stability in the banks and streambeds as well as securing that the channels have enough capacity to transport and utilize its sediment and debris effectively. However, an extensive understanding of how ecosystems have changed according to possible alternates of restoration projects (Parker, 1997; Beechie et al., 2000; Ehrenfeld, 2000) and deep considering of the major needs of the local communities (Ebersole and Liss, 1997) are important for the successful SSWRPs. Furthermore, changing of geographical features and channel shapes due to frequent local damages caused by flooding, sediment transport or urban development should be considered in the small stream restoration plans.

To reduce the flood damages in the small stream watersheds, Korea government legislated the Small Stream Development Act and has performed the SSWRPs since 1995. National Emergency Management Agency (NEMA), Korea established the master plan for the SSWRPs every year for the flood-related disaster risk reduction and resilient recovery. The main goals of watershed restoration projects includes rebuilding the channel banks or buying the land in the damaged areas along the stream. From 2005, small stream restoration project types are changing in order to cope with environmental changes as presented in Table 1. Despite the efforts done by the government for watershed restorations, numerous stream and watershed restoration projects failed to accomplish their all objectives excepting of flood damages reducing (Cheong et al., 2011). However, the small stream restoration plans have not represent recent extreme disasters caused by localized heavy rainfall, massive flash flooding and current social and environment changes. Each failures has varying causes such as : i) damages of the natural environment due to channel maintenance, aggregate extraction, floodplain development, ii) lack of animal and flora survey, iii) adverse effect on ecosystems caused by sediment outflow during restoration, iv) inconsistent small stream restoration between downstream and upstream areas, v) disconnection of ecosystem by submerged weir and drop structures, vi) simplification habitat for aquatic organisms and increased erosion due to the river straightening and riverbed flattening, vii) the lack of monitoring data such as water surface elevation and flow discharge for stream runoff analysis, viii) insufficiency on water pollution reduction measures, ix) the lack of guidelines for more appropriate budget allocation for the small stream watershed restoration projects, x) impaired flood control functions in downstream areas by basin or watershed units of a comprehensive disaster prevention restoration rather than prevention-oriented upstream restoration.

| Strategy | Periods | General Description |
| --- | --- | --- |
| Embankment Facility Restoration | 1995~1999 | - Focus on structural changes to prevent river bank - Lost original characteristics of natural streams by concrete embankment - Increase flood risk in downstream by changes in straight channel shape - Disconnect ecosystem by high cutting bank slope or retaining wall - Disharmonize with surrounding landscape by excessive changing channel pattern and geography |
| Landscape Facility Restoration | 2000~2004 | - Focus on development of environmental friendly natural stream - Prevent pool-riffle structures in the channel - Use eco-friendly material such as wood, grass, stone, clay and other natural mater-ials - Water as nature-friendly appearance, while very limited habitat features |
| Eco-river Facility Restoration | 2005~Now | - Use natural materials and prevent natural river sinuosity - Provide only part of the ecosystem as habitat - Lack of considering ecological characteristics of upper, middle and down parts in the small stream |

Table 1 : Brief description of three general strategies for prioritizing small stream restoration projects

Flood risk assessment results defined by priority ranking can be support for decision-making to allocate limited resources and select optimum small stream restoration projects fulfilling goals of SSWRPs and solve problems caused by many reasons listed above. The priority setting based on the flood risk assessment provides useful tool to reduce indiscriminate projects and lead effective watershed restoration projects for protecting sensitive watershed resources and reducing flood-related damages due to localized heavy rainfall and massive flash flooding. For example, if the small stream in mountainous or rural area has a low priority ranking on the flood risk assessment, building embankments to prevent high water from flooding the bordering land is sublated and diversity such as buying the agricultural lands along the stream to storage flood water is preferred for flood control on the small streams. The adaptable guideline is also needed for selecting more urgent or optimum restoration projects among various alternates of small stream watershed restoration projects submitted by local governments. It is also expected that the priority ranking can support the local government to establish optimal regional rehabilitation plans against flooding disasters with respect to the causes and characteristics of past floods and major needs of the local communities. Now, there are no flood related damages in small streams of urban areas because the restoration ratio of the small streams managed by metropolitan city government is almost 70%. Thus the guideline is needed to provide more appropriate budget allocation for the small stream watershed restoration projects to reduce the flood related damages in the small streams managed by local government and to increase flood related disaster management capacity of local government.

# Multi Criteria Decision Analysis Modelling on the SSWRPs

Multi Criteria Decision Analysis (MCDA) is aimed at supporting decision makers faced with evaluating alternatives taking into account multiple, and often conflictive, criteria. The MCDA process consists of the following phases: problem identification and structuring, model building and use; and the development of action plans. MCDA modelling consists of four key elements: (i) the alternatives to be appraised, (ii) the criteria (or attributes) against which the alternatives are appraised, (iii) scores that reflect the value of an alternative’s expected performance on the criteria, and (iv) criteria weights that measure the relative values of each criterion as compared to others. For the priority setting of the SSWRPs based on the flood risk assessment, a set of criteria was established to evaluate the needs of community, environmental effects and benefit of each small stream region. Evaluation criteria were determined by qualitative method to represent small stream characteristics such as i) reviewing existing river restoration plan developed by Ministry of Land, Infrastructure and Transport (MOLIT), ii) discussion with government experts, iii) deep consideration and iv) data availability. To minimize redundancy, some attributes such as two criteria should provide the same or very similar concept or measures excluding the objectives. MOLIT (1999) selected criteria such as i) the characteristics of the prone areas, ii) the frequency of the flood-related damages, iii) causes of damages and iv) the benefit to set the priority of the river restoration project on improving the flood prone area. MOLIT (2005) also developed additional guidelines in selected criteria such as regional characteristics, frequency of past flooding, flood risk analysis and economic dimensions (benefit-cost ratio). The evaluation criterion shown in Table 2 was used to determine the priorities of the river restoration projects according to the river master plans.

| Evaluation Criteria | Generalized Criterion |  |  |  |  | Weighting Factor |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 |  |
| Location | Mountainous | Agricultural | Rural | Small Urban | Urban | 10% |
| Completeness of Restoration | Complete | Almost | Half | a Little | Incomplete | 15% |
| Existing Facilities Condition | Very Good | Good | Fair | Poor | Very Poor | 15% |
| Future Planned Facilities | Nothing | a Few | Small | Many | So Many | 10% |
| Flooding Damages During Past 10yrs | 0 | 1~2 | 3~4 | 5~6 | > 7 | 20% |
| Civil Complaint During Past 10yrs | Nothing | a Few | Occasionally | Sometime | Frequent | 20% |
| Years after the river master plan | 10 Years | 12 Years | 13 Years | 14 Years | 15 Years | 10% |

Table 2 : The priority of the river restoration project on improving the flood prone area

To solve multi criteria problem on the SSWRPs priority setting based on the flood risk assessment, the 12 representative factors were carefully selected and constructed. Detailed hierarchical structure for priority setting for the flood risk assessment were categorized such as i) number of poor facilities, ii) number of disasters during past 5 years, iii) total damages during past 5 years, iv) type of damages, v) watershed area, vi) ratio of residential area to total watershed area, vii) channel slope, viii) restoration ratio of the restoration channel length to total channel length, ix) number of residents, x) waterfront revitalization area, xi) number of public facilities and xii) establishment of master plan. For criteria evaluation analysis, 212 small streams data were collected from all stream watersheds of Korea specifically 38 streams data from Gangwon-do, 65 streams data from Gyeongsang-Buk-do including 3 of Daegu, 41 streams data from Gyeongsang-Nam-do, 37 streams data from Jeonra-Buk-do and 30 streams data from Chungcheong-Buk-do. These small streams have length distribution between 650 m and 11,080 m with average length of 3,851 m. The statistical values such as the minimum, mean and maximum values of each criteria indicator collected in provinces were listed in Table 3. The 12 evaluation criteria were constructed for the three main aspects, such as PF (Pressure Factors), SF (State Factors), and RF (Response Factors including the capacities of local government) under the P-S-R (Pressure-State-Response) classification system for identifying the SSWRPs.

| Region | Statistics | C1 | C2 | C3 | C5 | C6 |  |  |  |  | C7 | C8 | C9 | C10 | C11 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Paddy | Farm | Forest | Site | Residential |  |  |  |  |  |  |
| Gang won-do | Mean | 15 | 2 | 0.1940 | 6.99 | 9.46 | 7.82 | 78.8 | 1.64 | 4.60 | 0.057 | 56.5 | 68 | 22.82 | 3 |
|  | Max. | 39 | 5 | 0.6880 | 30.9 | 25.2 | 14.1 | 90.0 | 5.20 | 8.90 | 0.286 | 100. | 482 | 171.5 | 8 |
|  | Min. | 1 | 1 | 0.0010 | 0.70 | 0.04 | 0.07 | 57.1 | 0.10 | 0.18 | 0.008 | 14.5 | 2 | 0.200 | 1 |
| Gyeong sang-Buk-do | Mean | 17 | 3 | 3.7200 | 7.28 | 15.9 | 6.85 | 71.0 | 3.25 | 4.85 | 0.061 | 32.5 | 84 | 16.97 | 6 |
|  | Max. | 64 | 8 | 25.790 | 41.1 | 58.9 | 16.9 | 96.3 | 15.1 | 15.6 | 0.330 | 80.0 | 570 | 66.25 | 17 |
|  | Min. | 1 | 1 | 0.0280 | 0.76 | 0.62 | 0.31 | 16.2 | 0.50 | 0.94 | 0.005 | 0.80 | 2 | 1.000 | 1 |
| Gyeong sang-Nam-do | Mean | 12 | 4 | 108.28 | 28.6 | 18.9 | 7.41 | 68.1 | 4.38 | 3.65 | 0.038 | 59.6 | 137 | 17.27 | 3 |
|  | Max. | 38 | 9 | 106.72 | 1028. | 42.5 | 18.3 | 89.1 | 12.6 | 7.80 | 0.142 | 100. | 1141 | 104.0 | 6 |
|  | Min. | 2 | 1 | 0.0023 | 0.39 | 3.70 | 0.60 | 33.7 | 0.52 | 0.10 | 0.001 | 0.7 | 17 | 0.210 | 1 |
| Jeonra-Buk-do | Mean | 11 | 3 | 106.32 | 3.24 | 19.0 | 8.19 | 62.3 | 4.79 | 8.09 | 0.056 | 67.2 | 121 | 52.57 | 9 |
|  | Max. | 29 | 7 | 655.29 | 9.95 | 50.0 | 27.7 | 86.2 | 19.9 | 26.4 | 0.228 | 100. | 795 | 211.0 | 47 |
|  | Min. | 1 | 2 | 0.1642 | 0.44 | 5.47 | 2.99 | 16.0 | 0.38 | 1.02 | 0.005 | 35.5 | 17 | 0.260 | 2 |
| Chungcheong-Buk-do | Mean | 16 | 4 | 0.1236 | 4.50 | 16.5 | 13.3 | 64.2 | 4.44 | 8.35 | 0.187 | 53.8 | 89 | 52.41 | 2 |
|  | Max. | 41 | 15 | 0.3583 | 11.3 | 43.0 | 39.6 | 82.0 | 8.76 | 40.0 | 0.883 | 100. | 292 | 296.0 | 6 |
|  | Min. | 3 | 1 | 0.0075 | 0.98 | 3.00 | 2.90 | 13.6 | 0.04 | 1.68 | 0.011 | 14.5 | 10 | 1.570 | 1 |
| Total | Mean | 19 | 4 | 10.416 | 66.2 | 22.5 | 11.8 | 58.9 | 6.01 | 8.81 | 0.135 | 54.4 | 255 | 67.61 | 8 |
|  | Max. | 64 | 15 | 106.72 | 1028. | 58.9 | 39.6 | 96.3 | 19.9 | 40.0 | 0.883 | 100. | 1141 | 296.0 | 47 |
|  | Min. | 1 | 1 | 0.0013 | 0.39 | 0.04 | 0.07 | 13.6 | 0.04 | 0.10 | 0.001 | 0.7 | 2 | 0.200 | 1 |

Table 3 : Statistical chart of criteria collected from small steam watersheds in Korea

This study used rating the relative priority of the criteria divided by five scale sections from 1 (equal importance) to 5 (extreme importance) to determine the priority ranking of the flood risk assessment for the SSWRPs. Each scale section was divided due to the normal distribution of each criteria for all the attributes of each alternate shown in Table 4. The weight was determined by entropy weight coefficient method followed by the procedure presented in Image 3 to determine the unbiased weight of the criteria. Entropy weight coefficient method, in which weights are determined by attributes information only to avoid distorted assessment by subjective opinion of decision-makers, can be used for multi-criteria decision-making cases including various attributes and many alternates. This method sets the attribute matrix of alternates to calculate entropy from normalized attributes and then determine the weight of criteria. The results of the comparison of weights of all four categories were determined to be 0.401, 0.299, 0.195 and 0.105 for the disaster risk as a Pressure Factors, watershed characteristics as a State Factors, restoration project benefits and establishment of master plan as a Response Factors, respectively. Based from the comparative results of the weights, factors regarding disaster risk were highly prioritized by small stream watershed restoration projects rather than environmental or ecological factors. The responses were checked with consistency index to obtain weight for each criterion. The response with consistency index of over the boundary were returned to the respondents for them to answer the questions. Evaluation criteria was classified to give high weight on the flood-prone areas or the areas with an urgent need for restoration and low weight on the small stream watershed areas located in the mountainous and rural regions. If the SSWRP was planned for buying farmland to use for the floodplain, connected to the national river, and is established for the flood risk reduction plan and, flood damage recovery, a high priority will be given.

| Evaluation Criteria |  |  | Generalized Criterion |  |  |  |  | Weight |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 | 2 | 3 | 4 | 5 |  |
| Pressure Factors | Number of Poor Facilities | C1 | ~1 | 2~10 | 11~20 | 20~30 | 31~ | 0.10 |
|  | Number of Disasters During Past 5 years | C2 | 0 | 1 | 2~3 | 4~5 | 6~ | 0.10 |
|  | Total Damages During Past 5yrs (M$) | C3 | 0 | 0.1~0.5 | 0.6~5 | 6~50 | 51~ | 0.10 |
|  | Type of Damages | C4 | Agricultural Land Inun-dation (AI) | Agricultural Land Lost (AL) | House Inundation (HI) | House Lost (HL) | Public Facilities Damages | 0.10 |
| State Factors | Watershed Area (km2) | C5 | 0~1.0 | 1.1~2.0 | 2.1~5.0 | 5.1~8.0 | 8.1~ | 0.05 |
|  | Residential Area Ratio1 | C6 | 0~20% | 21~40% | 41~60% | 61~80% | 81~100% | 0.15 |
|  | Channel Slope | C7 | ~0.100 | 0.101~0.070 | 0.071~0.040 | 0.041~0.010 | 0.011~0.000 | 0.05 |
|  | Restoration Ratio2 | C8 | 0~20% | 21~40% | 41~60% | 61~80% | 81~100% | 0.05 |
| Response Factors | Number of Residents | C9 | 0~10 | 11~40 | 41~80 | 81~160 | 161~ | 0.05 |
|  | Waterfront Revi-talization Area (ha) | C10 | 0~1.0 | 1.1~5.0 | 5.1~10.0 | 10.1~50.0 | 50.1~ | 0.10 |
|  | Number of Public Facilities | C11 | 0 | 1~2 | 3~4 | 5~7 | 8~ | 0.05 |
| Others | Establishment of Master Plan | C12 | - | Unfinished Master Plan (U) | Finished Master Plan (P) | Ongoing Project (O) | Last Year Project (L) | 0.10 |
| Total |  |  |  |  |  |  |  | 1.00 |

Table 4 : The indicators, sub indicators and criteria to set the priority of the SSWRPs ; 1Residential Area : Residential Area/Total Watershed Area; 2Restoration Ratio : Restoration Length/Total Channel Length

Image 3 : Concept diagram of the entropy weight calculation procedure

Comparisons of damages with implementation in small streams

Image 3 : Concept diagram of the entropy weight calculation procedure

Comparisons of damages with implementation in small streams

The objective of the Multi Criteria Decision Analysis (MCDA) is as follows:

(1)

where  is a finite set of  alternates and  to  are  criteria,  is the evaluation of alternate  on criterion . In order to achieve this objective, it is essential to have some information about the preferences and the priorities of the decision-maker. The two different MCDA approaches of the weighted method and the PROMETHEE approach are applied to demonstrate the potential advantages and pitfalls of using the different MCDA modelling approaches. A very common way to solve multi-criteria decision making is to compute a weighted sum of the evaluations as  where is the weight allocated to criterion, is the resulting score of alternate. This weighted method has several limits for the following reasons: i) bad evaluation on one criterion can always be compensated by better values on other criteria; ii) the weights of the criteria are linked to the scales of measurement of the criteria in a way that is difficult to manage; iii) for reducing an alternate to a single score, a lot of information should be lost about the conflicts among the criteria. For this reason Roy (1991) proposed to build outranking relations including only realistic enrichments of the dominance relation.

The PROMETHEE method is an outranking method based on the principle of pairwise comparison of the attributes. In this model, the weighting vector  represents the relative importance of criteria for the problem. The decision matrix  represents the performance ratings  of alternates  with respect to criteria. Given the weighting vector  and decision matrix, the objective is to rank or select the alternatives by giving each of them an overall preference value with respect to all criteria shown in Image 4. The decision maker perceives the measurement scale of the criterion in order to make a model, the PROMETHEE method, however, requires associating a preference function to each criterion as  in which  and for which. To maximize a criterion, this function is giving the preference of  over  for observed deviations between their evaluations on criterion. The six type of preference functions presented in Table 5 (Brans and Mareschal, 2005; Podvezko and Podviezko, 2009) can be used to predict priority ranking.

| Types | Function | Shape |
| --- | --- | --- |
| Type I (Usual) |  |  |
| Type II (U-shape) |  |  |
| Type III (V-shape) |  |  |
| Type IV (Discrete) |  |  |
| Type V (Linear) |  |  |
| Type VI (Gaussian) |  |  |

Table 5 : Six types of preference functions for the PROMETHEE method

Image 4 : Concept diagram of the PROMETHEE procedure

Comparisons of damages with implementation in small streams

Image 4 : Concept diagram of the PROMETHEE procedure

Comparisons of damages with implementation in small streams

For pairwise comparison of more than two criteria with given weight, preference indices are as follows:

(2)

(3)

where in the expression degree  is preferred to  over all the criteria and in   is preferred over . In most cases, there are criteria for which  is better than , and criteria for which  is better than . Consequently,  and  are usually positive. If   , a weak global preference of  over  is implied, and if a strong global preference of  over is implied. As each alternate  is facing  alternates in , the positive outranking flow,  and the negative outranking flow, , can be calculated by using the following equations:

(4)

(5)

The positive outranking flow expresses how an alternate is outranking the others. The PROMETHEE I use partial ranking  obtained from the positive and the negative outranking flows. are variables for preference, indifference and incomparability respectively.

(6)

(7)

(8)

For complete ranking, the PROMETHEE II uses the net outranking flow as. It is the balance between the positive and the negative outranking flows. The higher the net flow, the better the alternate, such as  and .  indicates that  is outranking all the alternates on all the criteria, however, if , others are more outranking than a. The complete ranking is easy to use, but the analysis of the incomparability often helps to finalize a proper decision. As the net flow  provides a complete ranking, it may be compared with a utility function. One advantage of  is that it is built on clear and simple preference information and that it relies on comparative statements rather than absolute ones.

# Results

# Official Priority Ranking Predicted by the Value Measurement Model

Official Priority Ranking Predicted by the Value Measurement ModeThe priority of alternatives for selecting optimum SSWRPs was determined by weighted method in which the priority ranking was calculated by multiplying the weight and scale section values determined from attributes of alternates. For the analysis, attribute information of 93 SSWRPs applied from local government in 2012 were collected which were specifically composed of 16 from Gangwon-do, 26 from Gyeongsang-Buk-do including 3 of Daegu, 25 from Gyeongsang-Nam-do, 11 from Jeonra-Buk-do and 15 from Chungcheong-Buk-do. The predicted results of priority ranking officially determined by the weighted method showed that A01 has the highest priority with net outranking flow of 0.4712 which mean that this stream is urgently needed for small stream watershed restoration followed by A13, A24 and A52, while A54 was the lowest priority with net outranking flow of -0.4296. Among the alternates, 42 small streams have positive net outranking flow and 51 small streams have negative net outranking flow. For comparison between the evaluation criteria and the net outranking flow values predicted by the weighted method, the alternates were divided into three groups: the upper rank group with the net outranking flow values from 0.2503 to 0.4712, the middle rank group with the net outranking flow values from 0.0108 to 0.2467 and the lower rank group with the net outranking flow values from -0.4296 to -0.0032 shown in Table 6. To define the main criteria affecting the priority setting of the SSWRPs, correlation coefficients of each criteria to the net outranking flow values were calculated and listed in Table 7. The results show that the number of poor facilities (C1), type of damages (C4) and establishment of master plan (C12) were significantly correlated with the priority setting in the upper and middle rank groups. In the lower group, the number of disasters during the past 5 years (C2), total damages during the past 5 years (C3), watershed area (C4) and restoration ratio (C8) were correlated with priority setting of small stream watershed restoration projects.

| Group | Net Outranking Flow | Alternatives |
| --- | --- | --- |
| Upper Rank | Positive (+) | A01, A13, A24, A52, A50, A30, A33, A22, A21, A18, A23, A35, A62, A55, A31 |
| Middle Rank | Positive (+) | A49, A87, A76, A93, A32, A70, A43, A26, A51, A46, A27, A28, A07, A08, A11, A69, A68, A59, A75, A72, A42, A80, A84, A64, A67, A65, A57 |
| Lower Rank | Negative (-) | A74, A88, A53, A73, A90, A82, A03, A47, A34, A02, A10, A16, A05, A81, A77, A78, A61, A85, A36, A25, A15, A45, A37, A17, A19, A40, A38, A04, A09, A39, A41, A89, A86, A92, A44, A48, A20, A29, A66, A56, A79, A06, A91, A58, A83, A71, A60, A63, A14, A12, A54 |

Table 6 : Categorization of alternates for comparison between the evaluation criteria and the net outranking flow values

| Group | Correlation Coefficient |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | C1 | C2 | C3 | C4 | C5 | C6 | C7 | C8 | C9 | C10 | C11 | C12 |
| Upper Rank | 0.632 | 0.284 | 0.363 | 0.483 | 0.054 | 0.325 | 0.019 | 0.270 | 0.291 | 0.408 | 0.239 | 0.615 |
| Middle Rank | 0.314 | 0.396 | 0.254 | 0.695 | 0.147 | 0.102 | 0.033 | 0.007 | 0.041 | 0.198 | 0.236 | 0.024 |
| Lower Rank | 0.145 | 0.443 | 0.435 | 0.420 | 0.096 | 0.008 | 0.150 | 0.495 | 0.003 | 0.398 | 0.181 | 0.162 |

Table 7 : The Correlation coefficient of each criteria to the net outranking flow values

# Priority Ranking Predicted by the Outranking Approach

This study used PROMETHEE to evaluate the given criteria in the process of determining the investment priorities, the positive outranking flow, , the negative outranking flow,  and the net outranking flow as  were calculated. The PROMETHEE uses preference indices for pairwise comparison of more than two criteria with given weight. Preference indices determined by Equations (2-3) with predetermined weights, preference functions and criteria were all calculated. For predicting priority ranking by the PROMETHEE, entropy weight coefficient method was also used to determine unbiased weight of the criteria. The estimation results of weights for the application of the PROMETHEE show that the residential area ratio (C6) had the highest value of 0.167 and the total damages during past 5yrs (C3) had 0.108 and both of the waterfront revitalization area (C10) and the establishment of master plan (C12) had 0.105. To determine the preference function of PROMETHEE, the characteristics of each attributes should be considered. The specific property of indicator consists of attributes represented by six types of distribution: Usual, U-shape, V-shape, Discrete, Linear and Gaussian distributions as shown in Table 5. To verify the present approach, the official priority ranking on the flood risk assessment for the SSWRPs from the weighted method are also compared with the values simulated with the PROMETHEE with six preference functions as shown in Image 5. The predicted ranking of the PROMETHEE with Gaussian preference function (Type 6) clearly predicts the official priority ranking. This is in contrast to the usual preference function (Type I) which seem to overestimate the official priority ranking. The PROMETHEE predictions with four types of preference functions (Type II-V) are also in good agreement with the priority ranking obtained by the weighted method. For comparison between the evaluation criteria and the net outranking flow values estimated by the PROMETHEE with Gaussian preference function, the net outranking flow values of 93 alternates were plotted as shown in Image 6. The results of priority ranking predicted by the PROMETHEE with Gaussian preference function show that A02 have the highest priority which mean that this stream is urgently needed for small stream watershed restoration followed by A11, A22 and A68, while A59 was the lowest priority as shown in Figure 6.

Image 5 : Comparisons of determined priority ranking based on flood risk assessment with official ranking data used in verification

Image 5 : Comparisons of determined priority ranking based on flood risk assessment with official ranking data used in verification

Image 6 : The results of the investment priorities, the positive outranking flow, the negative outranking flow and the net outranking flow

Image 6 : The results of the investment priorities, the positive outranking flow, the negative outranking flow and the net outranking flow

# Evaluations of Multi Criteria Decision Approaches on the SSWRPs

To evaluate the difference between the official and the predicted values of the flood risk assessment priority ranking more quantitatively, the discrepancy ratio defined by Cheong and Seo (2003) was used as measure of error.

(9)

in which  is the discrepancy ratio,  is  the predicted value and  is the official value. If the discrepancy ratio is 0, the predicted value is identical to the official values. If the discrepancy ratio is larger than 0, the predicted value is overestimated and if the discrepancy ratio is smaller than 0, the predicted value is underestimated Accuracy is defined as the proportion of numbers for which the discrepancy ratio is between -0.05 and 0.05 for the total number of data. Image 7 shows the histogram of discrepancy ratios for the proposed method used in the analysis of the 93 official priority ranking set. The discrepancy ratio distributions for the proposed method with Gaussian function presented a range of values between -0.3 and 0.3. The accuracy of the proposed method with the usual preference function was 35%, with the u-shape preference function as 60%, v-shape preference function as 57%, discrete preference function as 53%, and linear preference function as 61%, while that of the proposed method with the Gaussian preference function at 73%. Overall, the accuracy of the proposed method with the Gaussian preference function was the highest amongst all preference functions tested.

Image 7 : Comparison of discrepancy ratios of the PROMETHEE with six preference functions for official priority ranking used in verification

Image 7 : Comparison of discrepancy ratios of the PROMETHEE with six preference functions for official priority ranking used in verification

# Development of the Guideline for Selecting the Optimum SSWRPs

The small stream restoration projects are facing challenges regarding resources allocation and also selection of optimum measures on how to reduce flooding risk in their watershed areas. Flood risk assessment results defined by priority ranking of the SSWRPs could help decision-makers for planning future disaster risk reduction projects and to design future SSWRPs coping to extreme events such as flash floods and localized heavy rainfall. Efficiency of the priority setting of flood risk assessment could be evaluated by considering the following: i) whether priority setting has an improved capacity for making decision on the cost- and environment-effective small stream restoration measures; and ii) whether stakeholders and decision-makers felt that the priority setting process provided the needs of the community on resilient recovering and sustainable development. In this study, the optimal restoration project guideline was developed based on the priority ranking predicted by PROMETHEE with Gaussian preference function for decision-making in which alternates were divided into three ranking groups such as the upper rank, the middle rank and the lower rank as shown in Figure 6. To make a decision on the optimal SSWRPs, the decision-maker can select one management option among prevention or preparedness, sustainable development and resilient recovery by predicting the priority ranking and then determining the optimum restoration measure listed in Table 8. The guideline can be used to rebuild the small streams to be in harmony with the natural environment and to prepare floodplain to minimize future flood damages and to maximize natural stream habitat. This guideline can also help the government and the community for their decision-making regarding the protection of the sensitive watershed by means of floodplain or restoration of small stream watersheds for reducing flood risk. Reducing the embankment facility-oriented restoration project and increasing the landscape facility restoration project as well as the eco-river facility restoration project are some of the expected outcomes of this guideline.

| Group | Upper Rank | Middle Rank | Lower Rank |
| --- | --- | --- | --- |
| Management | Resilient Recovering | Sustainable Development | Prevention/Preparedness |
| Restoration Measures | Ecology, community stream | City, culture and theme stream | Channel and bank restoration |
|  | Purchase of lands for floodplain | Establishment of Artificial Park Area | Bridge Reconstruction |

Table 8 : Guideline for the restoration measures for the flood related disaster risk management

# Conclusions

Flood-related disasters on small streams caused by high intensity localized rainfall and flash floods has been continuously increasing especially for the residential areas. Recent survey results showed that the SSWRPs, as a one of the disaster risk reduction measures, could contribute to reduce flood-related damages in small streams. However, the existing restoration plans did not develop good responses in terms of extreme disasters and current social and environment changes due to geographical and physical features of stream channels like flooding, sediment transport or urban development. Furthermore, the SSWRPs were facing significant resource allocation challenges as well as problems for the selection of optimum measures for flood risk reduction considering community needs such as prevention or preparedness, sustainable development, resilient recovery and available resources. Flood risk assessment results defined by priority ranking can be used for decision-making to allocate resources and select optimum community based restoration measures. The 12 representative factors for the flood risk assessment are carefully selected and constructed for the three main aspects under the P-S-R classification system for identifying the 212 small stream watershed. The entropy weight coefficient method is also applied to calculate the weight of flood risk factors in order to reduce subjective judgments on the effects of the weight coefficients. The weighted method was used to predict the official priority ranking based on the flood risk assessment of 93 SSWRPs and the PROMETHEE with six preference functions was used here to validate predicted results with official values. To evaluate the practical application of the PROMETHEE for identifying the flood prone area, the results with six preference functions are compared with official priority ranking predicted by weighted method. The predicted priority ranking from the PROMETHEE with Gaussian preference function clearly predicts quite well the official priority ranking while the usual preference function overestimates the official values. The accuracy of the proposed method with the Gaussian preference function is the highest amongst all preference functions tested. Also optimal restoration guideline for decision-making such as prevention or preparedness, sustainable development and resilient recovering has been suggested by the determined three priority ranking groups. The guideline can be used to rebuild the small streams to be in harmony with the natural environment and to establish the floodplain to minimize future flood damages and to maximize natural stream habitat. It is expected that the flood risk assessment ranking ensured by full verification can support the optimal regional rehabilitation plans against flooding disasters with respect to the causes and characteristics of past floods. The procedures in this study can be used to establish evaluation criteria considering the weights in the decision-making process for similar projects using various alternates in small stream watershed restoration.

# References

Akter, T. and S.P. Simonovic. 2002. A general overview of multiobjective multiple-participant decision making for flood management. Water Resources Research Report No. 041. 65.

Bannerman, R.T., D.W. Owens, R.B. Dodds, and N.J. Hornewer. 1993. Sources of pollution in Wisconsin stormwater. Water Science & Technology, 28, 241-259.

Beechie, T.J., G. Pess, P. Kennard, R.E. Bilby and S. Bolton. 2000. Modeling recovery rates and pathways for woody debris recruitment in northwestern Washington streams. North American Journal of Fisheries Management. 20, 436–452.

Beechie, T.J. 2001. Empirical predictors of annual bed load travel distance, and implications for salmonid habitat restoration and protection. Earth Surface Processes and Landforms. 26, 1025–1034.

Brans, J.P. and B. Mareschal. 2005. PROMETHEE methods, in Figueira JR; Greco S; Ehrgott M (Eds.) Multiple Criteria Decision Analysis. State of the Art Surveys, Springer, Chapter 5. 163–195.

Bruen, M. 2002. Multiple criteria and decision support systems in water resources planning and river basin management. Proceedings of Irish National Hydrology Seminar, Irish National Committees. 25-35.

Cetinkaya, C.P., O. Fistikoglu, N.B. Harmancioglu and K. Fedra. 2007. Optimization methods applied for sustainable management of water-scarce basins. Journal of Hydroinformatics. 10(1), 69–95.

Cheong, E.S. 2011. Ranking Abnormality Occurs by Using the AHP Techniques. Journal of Korea Water Resources Association. 44(10), 72-79.

Cheong, T.S., B. Kang and S. Jeong. 2011. Study on Guideline for the Selection of Small Stream Implementation Projects. Journal of Korean Society of Hazard Mitigation. 11(2), 163-170.

Cheong, T.S. and I.S. Seo. 2003. Parameter Estimation of Transient Storage Model by Routing for River Mixing, Water Resources Research, 39(4), HWC 1-11.

Choi, S. 2009. Method to Evaluate Inundation Risk Using Multi Criteria Decision Making. MS Theses, Hanseo University.

Choi, H. I., S.Y. Park, J.H. Song and M.J. Park. 2013. Identification of Flood Risk Areas using a Multi-criteria Decision Making Method, Journal of Korean Society of Hazard Mitigation, 13(2), 237~243.

Coles, J.F., G. McMahon, A.H. Bell, L.R. Brown, F.A. Fitzpatrick, B.C. Eikenberry, M.D. Woodside, T.F. Cuffney, W.L. Bryant, K. Cappiella, L. Fraley-McNeal, and W.P. Stack. 2012. Effects of urban development on stream ecosystems in nine metropolitan study areas across the United States. Circular 1373. U.S. Geological Survey.

Collins, E. and L. Simpson. 2007. The Impact of Climate Change on insuring flood risk. Institute of Actuaries of Australia, New Zealand, 1-38

Ebersole, J.L. and W.J. Liss. 1997. Restoration of stream habitats in the western United States: restoration as reexpression of habitat capacity. Environmental Management. 21, 1–14.

Ehrenfeld, J.G. 2000. Defining the limits of restoration: the need for realistic goals. Restoration Ecology. 8, 2–9.

Gibson, J.L. 2004. Martin DK, Singer PA. Setting priorities in health care organizations: criteria, processes, and parameters of success. BMC Health Services Research. 4, 25:1-8.

Han River Flood Control Office. 2008. Hydrological annual report in Korea. Han River Flood Control Office, MLTM.

Hermans, C., J. Erickson, T. Noordewier, A. Sheldon and M. Kline. 2007. Collaborative environmental planning in river management: An application of multicriteria decision analysis in the White River Watershed in Vermont. Journal of Environmental Management. 84(4), 534-546.

Hong, S.J., Y.D. Lee, S.K. Kim and J.H. Kim. 2006. Evaluation of risk factors to detect anomaly in water supply networks- based on the PROMETHEE and ANP. Journal of Korea Water Resources Association. 39(1), 35-46.

Horboe, R. 1992. Multiobjective Decision making techniques for reservoir operation. Water Resources Bulletin. 28(1), 103-110.

Hyde, K.M., H.R. Maier and C.B. Colby. 2004. Reliability-based approach to multicriteria decision analysis for water resources. Journal of Water Resources Planning and Management. 130(6), 429-438.

Jaber, J.S. and M.S. Mohsen. 2001. Evaluation of non-conventional water resources supply in Jordan. Desalination. 136, 83-92.

Jandric, Z. and B. Srdjevic. 2000. Analytic hierarchy process in selecting best groundwater pond. 31st International Geological Congress. 6-17.

Jason, K.L., J. Hartmann, K.W. Li, Y. An and A. Asgary. 2007. Multi-criteria decision support systems for flood hazard mitigation and emergency response in urban watersheds. Journal of American Water Resources Association. 43(2), 346-358.

Kang, I. and Y. Yoon. 2002. A Study on the hydrologic decision-making for drought management : 2. decision-making method for drought management. Journal of Korea Water Resources Association. 35(5), 597-609.

Kang, M.G. and W.G. Kim. 2006. Use of index and indicator for assessment on water resources and environment in a watershed, Water and Future, 39(3), 37-48.

Khelifi, O., A. Lodolo, S. Vranes, G. Centi and S. Miertus. 2006. A web-based decision support tool for groundwater remediation technologies selection, Journal of Hydroinformatics. 8, 91-100

Kim, C. 2001. A Study on the decision-making for drought management using the drought indices. MS Theses, Inha University.

Kim, W. 2006. Determination of determination of investment priority for dams projects using multi criteria decision methods. PH.D. Theses, Chungnam National University.

Kim, G., C. Lee, G. Yeo and M. Shim. 2009. Priority decision of small hydropower development using spatial multi-criteria decision making. Journal of Korea Water Resources Association. 42(12), 1029-1038

Kim, G.H., S.P. Sun, K.D. Yeo and H.S. Kim. 2012. Determination of investment priority for river improvement project at downstream of dams using PROMETHEE. Journal of Korea Water Resources Association. 32(1B), 41-51.

Klein, R.D. 1979. Urbanization and stream impairment. Water Resources Bulletin. American Water Resources Association, 15, 948-963.

Ko, S.K., K.M. Kee and I.H. Ko. 1992. Comparative evaluation of multipurpose reservoir operating rules using multicriterion decision analysis techniques. Journal of Korea Water Resources Association. 25(1), 83-92.

Ko, S.K., D.G. Fontane and J. Margeta. 1994. Multiple reservoir system operational planning using multi-criterion decision analysis. European Journal of Operational Research. 76, 428-439

Korea Water Resources Corporation. 2002. A Study on water rights adjustments through the existing dam reasonable water allocation.

Lee, H. and M. Shim. 2002. Decision making for priority of water allocation during drought by analytic hierarchy Process. Journal of Korea Water Resources Association. 35(6), 703-714.

Lee, J.H., H.D. Jun, M.J. Park and J.H. Jung. 2011. Flash flood risk assessment using PROMETHEE and Entropy method. Journal of Korean Society of Hazard Mitigation. 11(3), 151-156.

Maragoudaki, R. and G. Tsakiris. 2005. Flood mitigation planning using PROMETHEE. European Water. 51-58.

Mitton, C.R. 2003. Donaldson C. Setting priorities and allocating resources in health regions: lessons from a project evaluating program budgeting and marginal analysis (PBMA). Health Policy. 64(3), 335-348.

MOLIT. 1999. Phase three basic plan report on prone area improvement project. Ministry of Land, Infrastructure and Transport, MOLIT.

MOLIT. 2005. Phase four basic plan report on prone area improvement project. Ministry of Land, Infrastructure and Transport, MOLIT.

Nam, D.S. 2007. Selection of flood control capability increase alternative for multi-purpose reservoir using multi-criteria decision analysis. MS Theses, Ajou University.

Min, W. 2009. Decision on priority of controlling non-point source pollution in upper stream of Soyang lake by using multicriteria decision making techniques. MS Theses, Seoul National University.

Park, T.S. 2002. The evaluation perspective of stream weight in AHP. Journal of Korea Water Resources Association. 35(6), 685-692.

Park, S.Y., J.H. Kim and S.K. Kim. 2005. A mixed-integer programming model to draw the concordance level and the kernel set for the implementation of ELECTRE IS. Journal of Korean Institute of Industrial Engineers. 31(4), 265-276.

Parker, V.T. 1997. The scale of successional models and restoration objectives. Restoration Ecology. 5, 301–306.

Pess, G., S. Morley and P. Roni. 2005. Evaluating fish response to culver replacement and other methods for reconnecting isolated aquatic habitats. American Fisheries Society. 267–276

Podvezko, V. and A. Podviezko. 2009. PROMETHEE I method application for identification of the best alternative, Business. Theory and Practice. 10(2), 84–92.

Raju, K.S., L. Ducjstein and C. Arondel. 2000. Multicriterion analysis for sustainable water resources planning: A case study in Spain. Water Resources Management. 14(6), 435-456.

Ridgley, M.A. 1992. Selection of water-supply projects under drought. Journal of Environmental Systems. 21(3), 207-221.

Ridgley, M.A. 1993. A multicriteria approach to allocating water during drought. Resource Management and Optimization. 9(2), 135-149.

Roni, P., T.J. Beechie, R.E. Bilby, F.E. Leonetti, M.M. Pollock and G.P. Pess. 2002. A review of stream restoration techniques and a hierarchical strategy for prioritizing restoration in Pacific Northwest watersheds. North American Journal of Fisheries Management. 22, 1–20.

Roy, B. 1991. The outranking approach and the foundation of ELECTRE Method. Theory and Decision. 31, 49-73.

Shafike, N.G., L. Duckstein and T. Maddock. 1992. Multicriterion Analysis of Groundwater Contamination Management. Water Resources Bulletin. 28(1), 33-42.

Udías, A., L. Galbiati, F.J. Elorza, R. Efremov, J. Pons and G. Borras. 2011. Framework for multi-criteria decision management in watershed restoration. Journal of Hydroinformatics. 14(2), 395–411

Woldt, W. and I. Bogardi. 1992. Ground water monitoring network design using multiple criteria decision making and geostatistics. Water Resources Bulletin. 28(1), 45-62.